* Design for Torsion

| * Project : | سقف المتكرر |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Concrete Fcu = | 250 | kg/cm2 |  |  |  |  |  |
| Concrete qall = | 9.682 | kg/cm2 |  |  |  |  |  |
| Stirrups Fy = | 2400 | kg/cm2 |  |  |  |  |  |
| Horizontal bars Fy = | 3600 | kg/cm2 |  |  |  |  |  |

|  | Ult. torsional moment |  | Sec. dim. |  |  | Ult. shear force |
| --- | --- | --- | --- | --- | --- | --- |
| Sec. | Mu (m.t) |  | b (cm) | t (cm) |  | Qu (ton) |
| 1 | 2.4 |  | 30 | 70 |  | 3 |

| Due to shear |  | qsu |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  | 1.54 | kg/cm2 |  | safe |
| Due to torsion |  | qtu |  |  | Notes |
|  |  | 12.86 | kg/cm2 |  | safe |

| Calculation of stirrups: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Due to shear |  | no. |  |  | Notes |
|  |  | 1.4 | 8 |  | 58/m` |
| Due to torsion |  | no. |  |  | Notes |
|  |  | 5.6 | 8 |  | 68/m` |

* Check shear in beams

| Concrete Fcu = |  | 250 |  | kg/cm2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Concrete qall = |  | 9.682 |  | kg/cm2 |  |  |  |  |  |  |  |  |  |
| Stirrups Fy = |  | 2400 |  | kg/cm2 |  |  |  |  |  |  |  |  |  |
| sac | Ult. Shear |  | Breadth |  | Depth |  | qu | As | no. of | Stirrups |  |  | Notes |
|  | Qu (ton) |  | b (cm) |  | d (cm) |  | (kg/cm2) | S | bran. |  |  |  |  |
| 1 | 16 |  | 25 |  | 65 |  | 9.85 | 0.060 | 2 | 6 |  | 8 | safe |
| 2 | 7.5 |  | 25 |  | 65 |  | 4.62 | -0. 03 | 2 | 5 |  | 8 | safe |
| 3 | 7.155 |  | 12 |  | 65 |  | 9.17 | 0.025 | 2 | 5 |  | 8 | safe |
| 4 | 11.4 |  | 12 |  | 65 |  | 14.62 | 0.056 | 2 | 6 |  | 8 | safe |
| 5 | 5.745 |  | 12 |  | 45 |  | 10.64 | 0.033 | 2 | 5 |  | 8 | safe |