Fundamentals of

Reinforced Concrete Design

ENCE335

Dr. Mirvat Bulbul

Room E408

2011-2012

Textbook: Reinforced Concrete Design, Chu-Kia Wang, Charles Salmon, Jose Pincheria, 7th Edition, John Wiley & Sons, 2007

| Introduction | All | Ch1 |
| --- | --- | --- |
| Design methods and requirements | 2.1-2.7 | Ch2 |
| Flexure in rectangular beams | 3.1-3.11 | Ch3 |
| Deflections under service loads | 4.8 | Ch4 |
| Self-study chapter +ACI coefficients | All+ 7.4 | Ch7 |
| One-way slab | All | Ch8 |
| T-beams | All | Ch9 |
| Self-study | All | Ch10 |
| Shear design | 5.1-5.13 | Ch5 |
| Development of Rebar | All | Ch6 |

ACI CODE

| Topics | Clauses |
| --- | --- |
| Details of Reinforcement | 7.1, 7.2, 7.6.1- 7.6.6 7.7.1, 7.7.4, 7.11, 7.12, 7.13 |
| Strength and Serviceability Requirement | 9.1- 9.3, 9.5.1, 9.5.2 |
| Flexure and Axial Loads | 10.2-10.6 |
| Shear and torsion | 11.1, 11.3, 11.5 |
| Development and splices of reinforcement | 12.1 - 12.5, 12.10 - 12.12, 12.15, 12.16 |

Concrete

- Plain Concrete is made by mixing certain proportions of cement, water, aggregates and other additives into a workable mixture (mix-design).

- In its plastic form (before setting), it can be cast into any form.  Hardened concrete is strong in compression, fire-resistant and durable.

- However, it is a non-structural material because it has no tensile strength and exhibits a brittle behavior.

- Strength of concrete is influenced primarily by w/c ratio. Other factors include compaction, curing, temperature, time, etc.

- Creep strain is dependent primarily on the intensity of the sustained load and is proportional to the logarithm of time under load. It results in long-term deflection in beams 2-3 times initial deflections. It is beneficial in redistribution of stresses by relieving high local stress concentrations that may cause failure.

- Shrinkage strain is the shortening per unit length associated with reduction in volume due to moisture loss. It is a function of water content, surrounding humidity and the surface/volume of the concrete.

- Differential drying set up differential stresses within the element. Also, if element is restrained then additional tensile stresses are set up which may lead to cracking.

- These can be limited by

  - minimizing water content

  - curing

  - limiting area/size of the pour

  - use of construction/expansion joints

  - shrinkage steel (a well-distributed grid of bars) can reduce size of cracks

BRITTLE: cannot undergo large deformations under load and fails suddenly without warning.

Maximum Compressive Strength, f'c

It is determined from a uniaxial compression test of cylinder (6inches in diameter, 12inches long) crushed at 28 days after casting and curing.

Cubes (150mm) are also used. Lower values of compression strength result from cylinder tests since the mid-part of the specimen is completely free from any restraint from the platens of the testing machine.  The ratio of the two tests varies from 0.81-0.96 as the cube strength varies from 25-52MPa.

Other Design Parameters

The secant modulus of elasticity (nonlinear -),

Ec=wc1.5(0.043)f'cMPa 	for wc=1440−2480kgm3 (cl.8.5.1)

For normal weight concrete, Ec=4700f'cMPa

Poisson's ratio  = 0.2

Ultimate strain at which total failure occurs,

 = 0.003 (cl. 10.2.3).

Tensile Strength (cl.9.5.2.3)

Tests include the split-cylinder test and the standard beam test.

The tensile strength, modulus of rupture, is variable and is approximated at 8-15%f'c,

fr=0.62f'cMPa

Steel Reinforcement

- Steel has high strength, ductility and stiffness, but suffers from susceptibility to corrosion and loss of strength at high temperatures (600oC).

- The idealized stress-strain diagram for steel rebar includes linear elastic region and a perfectly plastic (yielding) plateau.

- Steel with varying yield stress fy is available in 3 grades, namely: (40-60-75ksi equivalent to 276-414-517 MPa).

- Most common is fy= 60ksi = 414MPa.

- Modulus of Elasticity for steel Es= 200,000MPa

DUCTILE: undergoes large deformations under load and gives ample warning before failure.

- Deformed bars are used in reinforced concrete to improve the bond between the two materials. They are specified by their bar numbers (ACI) or their bar diameter (BS).

| Bar No. | Diameter (mm) | Bar No. |  Diameter (mm) |
| --- | --- | --- | --- |
| 3 | 10 | 8 | 25 |
| 4 | 12 | 9 | 28 |
| 5 | 16 | 10 | 32 |
| 6 | 20 | 11* | 36 |
| 7 | 22 | 14* | 43 |

Reinforced Concrete

- Reinforced Concrete combines both materials by improving their behavior so that the resulting composite material can resist both tension and compression, has a fire-resistance and a ductile behavior.

- This limits the possibility of progressive collapse in which a local failure spreads to the entire structure or to a significant portion of the structure.

- The ductility in reinforced concrete structures is achieved by (cl.7.13)

- Continuity of rebar between members

- Providing effective anchorage of rebar

- American Concrete Institute (ACI) Building Code provides technical specifications for design and construction of concrete buildings. The ACI employs empirical means to estimate the true behavior of reinforced concrete. Variations from the code are only allowed if sufficient testing and analysis can be established.

Empirical: Design based on experimental tests and experience rather than on theoretical formulations exclusively.

Reinforced Concrete Design

Loading

Service Loads vary depending on the structure in question and are classified as gravity and lateral loads.

Gravity loads include

- Dead Loads: concrete is a heavy material and its self-weight cannot be ignored. In design, there are some rules of thump for initial sizing of the member dimensions (preliminary design). Dead loads also include finishes and permanent walls and partitions.

- Live Loads are associated with building use/function and are specified in codes of practice.

| Use | Load (KN/m2) |
| --- | --- |
| Private Flats | 2.0-2.5 |
| Stores/offices | 3.5-5.0 |
| warehouses | 6.0-12.0 |

Lateral loads include:

- Wind (for tall buildings)

- Earthquake loading in seismic zones and others.

Design Philosophy

Ultimate strength design or strength design is the approach adopted in the design of concrete element.

Members are sized for factored loads (ultimate loads) obtained by multiplying service loads with load factors.

Elastic analysis of the structure for a variety of load combinations is undertaken depending on the load to which the structure is subjected. The required strength of a member corresponds to the most critical load combination.

Load combinations and the required load factors are defined by the ACI code 2005 under cl.9.2.

Examples include:

U 	= 1.2D+1.6L

U	= 1.2D+1.0L+1.6W

U	= 0.9D+1.6W

Note: The load factors of older code version used from 1971 until 1999 are included in Appendix C of the current code with limitations on their use.

Nominal strength of a member is obtained from the state of stress associated with a particular mode of failure. In order to account for imperfections, the nominal strength is reduced by a capacity reduction factor, . Hence,

Design strength ≤  (Nominal strength)

The value of  is influenced by the ductility of the member, accuracy of capacity prediction and importance of the member in the overall structure. (cl.9.3.2).

| Nominal strength |  |
| --- | --- |
| pure flexure | 0.9 |
| Shear and torsion | 0.75 |
| Spiral columns | 0.7 |
| Tied columns | 0.65 |

The extra capacity not only provides a factor of safety against failure (strength criteria) but also limits the service stresses to control deflection, and cracking (serviceability criteria). This approach is based on predicting the failure load rather than the actual stresses at service loads.

Serviceability: A structure should serve its intended purpose without excessive deformations, cracking, vibrations that may render the structure inadequate.

The latter approach is called Elastic Design. It does not take into consideration failure modes, initial stresses (shrinkage), redistribution of stresses (creep) and the reserve strength to failure. This method is limited now to design of fluid-retaining structures where low stress levels are desirable to limit crack widths.

Working-Stress Design (Elastic Design) is based on service loads and restricts elements stresses below an allowable stress set at some fraction of the failure stress.

Flexural Design

Three levels of loading to be considered:

Level 1 Uncracked Cross-Section

Assumptions:

- Plane sections before bending remain plane after bending i.e. linear strain distribution.

- Linear elastic behavior, Hooke’s law applies

- Maximum tensile stress ≤ fr , hence the gross cross-section is considered.

- Only minimum area of flexural reinforcement is provided and is ignored in the calculations.

- fr=McryIgDraw the strains, stresses and equivalent forces in the cross-section a below and show that the neutral axis passes through the centroid of the cross-section and the cracking moment can be evaluated from

Level 2 Service Load - cracked Section

Assumptions

- Plane sections before bending remain plane after bending i.e. linear strain distribution.

- Linear elastic behavior, Hooke’s law applies

- Concrete in tension section fully cracked

- No slip between steel and concrete

Singly-Reinforced Rectangular Beam

- Draw strains, stresses and equivalent forces in the cross-section to set up the following equations:

Equilibrium equation are given by:

12cbfc=Asfs  	                       C=T

12cbEcεc=AsEsεs

εcc=εsd−cand compatibility equation

n=EsEcIn order to find the depth of the neutral axis, we define the modular ratio, n, as

ρ=Asbdand the reinforcement ratio, , as

cd=−ρn+(ρn)2+2ρnthen

Alternatively, use the transformed-area of steel and apply the flexure formula on the transformed section (as in Mechanics of Materials). The N.A. will pass through the centroid of the transformed section.

Bending moment capacity is given by

M=AsfslaM=12cfclala=(d−c3)where la is the lever arm.

From the above two equations, the actual service stresses, fc and fs in concrete and steel can be calculated.

- Draw the strains, stresses and forces distribution in the transformed section.

Doubly Reinforced Beams

In a similar manner, the transformed area of the compression steel should be multiplied by (n-1) instead to account for the concrete.

- Draw the strains, stresses and equivalent forces are shown below and set up the equilibrium and compatibility equations

Control of Deflections

- Where deflections are not computed, ACI code places restrictions on the minimum depth of the flexure member (Table 9.5a):

|  | Minimum depth |  |  |  |
| --- | --- | --- | --- | --- |
|  | Simply supported | One end continuous | Both ends continuous | Cantilever |
| One-way solid slab | l/20 | l/24 | l/28 | l/10 |
| Beam and one-way ribbed slabs | l/16 | l/18.5 | l/21 | l/8 |

Notes:

Values given are for normal weight concrete and Grade 420 reinforcement. For other conditions, the values shall be modified as follows:

    - For structural light weight concrete having unit density, wc, in the range

1440-1920 kg/m3, the values are multiplied by (1.65 – 0.003wc)  1.09

    - For fy other than 420 MPa, the values are multiplied by (0.4 + fy /700)

Computing Deflections

Where deflections are computed, their values must not exceed the limits specified in Table (9.5b) (self study)

The effective moment of inertia, Ie, is defined depending on the case of loading (cl.9.5.2.3)

Case I 			 Ma /Mcr ≤ 1

Ie = Ig

Case II			1≤ Ma /Mcr ≤ 3

Ie=McrMa3Ig+1−McrMa3Icr≤Ig

where Ma is maximum moment in the beam at service load.

Additional long-term deflection resulting from creep and shrinkage of flexural members shall be determined by multiplying the part of the immediate deflection (elastic) caused by the sustained load, by the following factor

λ=ζ1+50ρ'

where ρ′ is the value for the compression steel at mid-span for simple and continuous spans, and at support for cantilevers,

and ξ  is time-dependent factor for sustained loads (values 1.0 - 2.0)

Level 3 Nominal strength: Moment capacity at failure

Three modes of bending failure are possible depending on the tensile steel percentage within the beam cross-section.

Case I (Over-reinforced beam or compression-controlled section)

- The brittle failure is initiated by the crushing of the concrete (at failure compression strain, εcu = 0.003), by a sudden disintegration of the compression zone

- At failure, deflections are still small with no extensive cracks in the tension zone.

- The strain in the tension steel does not reach yield, εt ≤ εy

- This is an uneconomic section since the capacity of the steel is not utilized and this behavior is not allowed by ACI code.

Case II (Under-reinforced beam or tension-controlled section)

- The ductile failure is initiated by the yielding of steel while strains in the concrete are still low.

- The beam exhibits large deformations while continuing to support loads up to collapse.

- Total failure is assumed when compression strain of 0.003 is reached in outermost concrete fibers while the strain in the tension steel is in excess of the yield strain.

- ACI flexural design is based on this behavior.

Case III (Lightly-reinforced beam)

- These are beams supporting loads well below Mcr, which is about 15-20% of Mn

- The brittle failure mode occurs when the tensile strength in the concrete exceeds the modulus of rupture and cracking starts releasing tensile stresses that the light steel reinforcement cannot absorb.

- The steel snaps and a total rupture of the beam follow.

- This behavior is not allowed by ACI and is controlled by specifying a minimum area of steel in the cross-section.

Minimum Reinforcement Ratio

Two limits are specified in the code; the first is related to the steel strength and the second to the concrete strength as follows:

  - When flexure produces compression in the flange (cl. 10.5.1)

ρmin=f'c4fy≥1.4fy

As,min=ρminbwd

where bw = width  of the web (mm) and d = effective depth (mm)

  - When flexure produces tension in the flange which lacks the ability to redistribute moment (as in determinate system), then As,min must be equal to or greater than the smaller of (cl.10.5.2)

As,min=f'c4fy(2bw)d

As.min=f'c4fybfd≥1.4fybfd

where bf = width of the tension flange

  - The above limits can be ignored if the steel provided is at least 1/3 greater than that required by the analysis (cl.10.5.2)

Strength Design of Rectangular Beams

The variation of concrete stress in the compression zone between the neutral axis and the extreme compression fiber is nonlinear which complicates the derivation of the design equation.

The state of stress is approximated by an equivalent Whitney rectangular stress distribution with an average compressive stress of 0.85f'c, acting over a depth of a =β1c. The equivalent block produces a resultant internal compression force that closely approximates the magnitude and line of action of actual resultant developed in the cross-section under loading.

Whitney determined that β1 =0.85 for f’c ≤ 30Mpa and reduces by 0.05 for each 7Mpa of f’c in excess of 30Mpa, but not less than 0.65.

- Draw the strains, stresses and equivalent forces at failure.

From equilibrium,

C=TC=o.85f'cbaT=AsfyMn=C(d−a2)=T(d−a2)

Alternatively, solving for a from the first equation and replacing its expression in the moment equation yields

Mn=Rnbd2Rn=fyρ1−ρm2m=fy0.85f'cρ=Asbd

Balanced Failure of a Beam occurs when the strain in tension steel reaches yield as the strain in outermost fibers of concrete reach a compression strain of 0.003.

- Draw the strains, stresses and equivalent forces at balanced conditions:

From compatibility requirements

0.003cb=εyd−cb

Solving for cb and replacing

εy=fyEs where Es =200GPa yields

cb=600600+fyd

For a rectangular beam, setting up an equilibrium equation and solving for the steel ratio that leads to balanced failure

ρb=Asbbd=0.85β1f'cfy600600+fy

To find the balanced steel for beams with non-rectangular compression zone, the same procedure is followed taking into consideration the compression area.

The balanced condition is given by

Asb=Cfy

Maximum tension steel

Similar to the balanced reinforcement ratio, there is a unique amount of rebar that cause the tension steel to reach minimum net tensile strain of 0.004 to ensure ductile failure, ACI code (10.3.3) requires

ρmax=0.003+εy0.007ρb

For fy =400MPa, max = 0.72b

One-Way Solid Slabs

These are structural members whose width and length are large compared to their thickness. Their thickness is usually controlled by deflection limitations.  They are designed as individual beams of 1m-width.

Tension Steel ratio is usually closer to the minimum limit. Rebar is specified by diameter size and the number of bars in 1m width or the bar spacing.

Shrinkage Steel

Although one-way slabs are assumed to bend in one-direction, rebar must also be placed in the lateral direction (perpendicular to the main tension steel) to limit cracking due to thermal and shrinkage stresses.

Shrinkage steel is specified using empirical equations (cl.7.12)

AsAg=0.0018(400)fy≥0.0014   for fy ≥ 400

Bar spacing is not to exceed 450mm or five times the slab thickness.

Flanged Beams

They occur when beams are poured monolithically with the slab. Most efficiently used when the flange is in compression producing a large compression force, developed at a larger lever arm.

They are lighter sections than their rectangular counterpart because the cracked concrete in the tension zone is eliminated (especially for long-span beams).

Effective width of the flange, bE (cl. 8.10)

T-beam: the smaller of

- l/4 beam span length

- bw+ 16 hf (8 hf either side of web)

- center-to-center spacing of beams

L-beam: the smaller of

- bw +l/12 beam span length

- bw+ 6 hf

- bw+ 1/2 ln (clear distance to next beam)

Also	bE ≤ 4bw  and hf  ≥ 1/2 bw

Flange Steel Distribution

Longitudinal steel: Tension steel is to be distributed in the effective flange width provided it is less than 10% of beam span. Otherwise, additional steel is to be provided outside this zone.

Transverse steel: These are placed at the top of the overhang and sized as for a cantilever fixed at face of web. (cl.8.10.5)

Flange-Beam Design

Two cases are considered:

Case I: N.A. within flange depth (a  hf)

- Draw strains, stresses and equivalent forces for the beam:

Design as for rectangular beam with b =bf

Case II: N.A. within beam web (a > hf)

- Draw strains, stresses and equivalent forces for the beam and set up the equilibrium and compatibility equations:

Mn=M1+M2

M1=0.85f'cAf(d−hf2)M2=0.85f'cAw(d−a2)

where		Af = (bf - bw) d

Aw= bw a

As = Asf + Asw

- Now calculate the balanced steel area for the T-section from first principles especially when N.A at balanced conditions fall outside the flange.

Doubly Reinforced Beams

In singly-reinforced Beams, Maximum moment capacity developed by a section is achieved when ρ=ρmax.

Where this moment is insufficient and beam dimensions cannot be increased, additional bending strength is achieved by adding additional steel in the tension and compression zones of the beam.

Example: doubly reinforced section to meet the highest  negative moment in a continuous T-beam at first support.

Note that the beam's behavior remains ductile because the balanced steel conditions is increased (see later)

Compression steel improves beam behavior by raising the amount of compressive strain in concrete before crushing, reducing creep and increasing ductility. This is the reason for minimum requirement of compression steel in all beams especially in seismic zones.

In order to improve confinement of concrete, all compression steel must be enclosed by lateral ties in accordance with (cl.7.11.1, cl.7.10.5.2)

Balanced Steel in Doubly-reinforced sections

- Draw the strains, stresses and equivalent forces in the beam at balanced conditions:

Compatibility relations:

cb=600600+fyd0.003cb=ε'sbcb−d'

We should investigate whether the compression steel has yielded or not based on the strain diagram relations above.

If compression steel has not yielded, then

ε'sb ≤ εy     and     f'sb ='sbEs

otherwise

f'sb =  fy

Equilibrium equations,

T=Cc+CsAsb=Cc+Csfy

For ductile behavior, ACI requires that εt at nominal strength shall not be less than 0.004 (cl.10.3.5)

Design of Doubly-Reinforced

Rectangular Beams (ductile failure)

In this design, there are more unknowns than equations: depth of the N.A., and the stress in the compression steel (yielded or not) and hence the solution proceeds incrementally (trial and adjustment) updating the value of c until equilibrium (convergence) is achieved.

- Draw the strains, stresses and equivalent forces diagrams:

The equilibrium equations are:

T=Cc+CsAsfy=0.85f'cba+A'sf's

Mn=C's(d−d')+Cc(d−a2)

The compatibility checks are:

Simplified Analysis of Continuous Beams (cl.8.3)

The following approximation of moments and shear envelopes (corresponding to maximum values of a number of load cases and combinations as specified by ACI) shall be permitted in the design of continuous beams and one way slabs provided:

        - Minimum of 2 spans

        - Approximately equal spans where the longer of any 2 adjacent spans is not more than 20% of the shorter span.

        - Uniformly distributed load only

        - Live load does not exceed 3 times the dead load

        - Prismatic members only

| Internal force | End span |  |  | Interior span |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End support | Midspan | First Support |  | Mid-span | Internal Support |
| Positive moment |  | (s.s) wuln2/11 (restrained)wuln2/14 |  |  | wuln2/16 |  |
| Negative Moment | (slab) wuln2/24 (column)wuln2/16 |  | (2spans)wuln2/9 (>2) wuln2/10 |  |  | wuln2/11 |
| Shear force | 1.15wuln/2 |  | wuln/2 |  |  | wuln/2 |

Shear and Diagonal Tension

Lateral loading and variation in bending along the beams produce shear forces in the beams. These forces are usually less significant than bending so beams are proportioned to resist flexure and only checked for shear. Pure shear induces tensile stresses on diagonal planes at 45o to the plane on which the maximum shear stresses are acting. These are known as the principle tensile stresses. Cracks develop perpendicular to this plane.

Since concrete is weak in tension, this causes brittle failure in beams unless shear reinforcement such as lateral ties (stirrups or links) or bent-up bars are provided. Hence, greater safety factor is required against this type of failure and ф = 0.75

Behavior of beams without shear reinforcement

The transfer of shear in reinforced concrete members without shear reinforcement occurs by a combination of the following mechanism:

- Shear resistance of the uncracked concrete;

- Aggregate interlock

- Dowel action

The following modes of failure are typical depending on the slenderness of the beam. Slenderness of a beam is defined by the ratio of its shear span to effective depth. For distributed loads, the shear span is the clear beam span (lc) while for point load the shear span (a) is the minimum distance from the point load to face of support.

    - Slender/long beams

(a/d > 5.5 )

Failure is initiated by the development of vertical cracks in the tension zone at middle third of span start at about 50% of failure load. As load increases, additional cracks spread and initial cracks widen and extend to N.A. and beyond with increased deflection of the beam leading to a ductile flexure failure in under-reinforced beams.

    - Intermediate slenderness

(2.5 < a/d< 6.0)

Cracks start as fine vertical flexural cracks at mid-span that never reach N.A. This is followed by the destruction of bond         between reinforcing tension steel and surrounding concrete near support. Two or three diagonal cracks develop suddenly at 1.5d-2d distance from face of support. One of them widens and extends to the top of the compression fibers of the beam causing a brittle failure with 	relatively small beam deflection. This is known as diagonal tension failure which characterizes the beam design.

    - Shear Compression Failure (deep beams)

(a/d < 2.5)

Few fine vertical flexural cracks start at mid-span but never reach 	N.A., followed by destruction of bond between reinforcing steel 	and surrounding concrete near support. A steeper diagonal crack 	develop suddenly at 1.5d-2d distance from face of support and 	propagate to N.A. to be arrested by the crushing of the concrete in 	the top compression fiber leading to re-distribution of stress. 	Brittle failure occurs when diagonal crack joins crushed concrete. 	This is known as shear compression failure.

Design Procedure for Shear

Remember that

- Concrete is a heterogeneous material that does not exhibit a linear elastic behavior in compression.

- It has little tensile strength of high variability.

- The cracked cross-section is variable along the beam span

- Shear failure corresponds to a diagonal crack

Hence, Mechanics of Materials approach to evaluate shear stress using ν=VQIb cannot be applied.

Instead an empirical approach is used to simplify the problem based on the following assumptions:

- Shear failure at a particular section occurs on a vertical plane when shear force exceeds concrete's fictitious vertical shear strength.

- Using experiments, fictitious vertical shear strength is related to f'c and properties of cross-section.

- By limiting stress values and specifying critical planes, brittle failure modes are eliminated.

- The average ultimate shear stress on a cross-section is approximated by

νu=Vbwd  MPa

where  V = the ultimate shear force on the cross-section in (N);

bw and d are the width of beam web and effective depth of the beam respectively in (mm);

Note: the value calculated (u) is significantly lower than the actual maximum shear stress but this has been taken into account by reducing the nominal strength of the concrete in shear (c).

Shear Strength of Concrete, c

Factors:

- Since shear failure is initiated by diagonal tension then it is expected that c is a function of fc'(as in modulus of rupture) and not fc (compression strength)

- Moment determines the intensity of cracking in the section and the cross-section available to resist applied shear.

- Length and width of crack are reduced as  (longitudinal steel) increases and some shear is carried by dowel-action in the steel.

- Short deep beam loaded from top has higher shear capacity than beams of moderate depth. Therefore, the design of shear does not apply to deep beams.

- Axial load acting simultaneously with shear on a cross-section modifies available shear capacity. Axial tension uses a proportion of the tension capacity while axial compression decrease the diagonal tension created by shear, hence raising its shear capacity.

For Normal weight concrete, the following equations apply (cl.11.3):

Case I: Combined bending and shear

νc=0.16fc'+17ρwVudMu≤0.29fc'MPa,               where  VudMu≤1

Where Mu is the factored moment occurring simultaneously with Vu for which shear strength is being provided.

Case II: Combined axial force and shear

In the presence of compressive axial force, shear capacity is increased to

νc=0.17(1+Nu14Ag)fc' MPa, where Nu in (N) and Ag in (mm2)

In the presence of tensile axial force, shear capacity is reduced			νc=0.17(1+0.29NuAg)fc' MPa, such that Nu < 0

For values of c ≤ 0, shear reinforcement is provided to carry the total value of Vu.

Case III: A simplified but conservative estimate for flexure and shear

νc=0.17fc'MPa

ACI code requires all flexural elements to have shear reinforcement to

- limit growth of diagonal cracks

- provide ductility

- prevent complete rupture if diagonal crack forms

- closely spaced stirrups hold longitudinal tension steel from tearing through concrete cover and prevent slipping.

No shear reinforcements are required: (cl.11.5.6)

- in beams when the design shear force satisfies

Vu≤φVc2

- in shallow members (e.g. slabs, footing and floor joists, ribbed slab)  where

h ≤ 250mm

h ≤ 2.5 hf

h ≤ 0.5 bw

when	Vu≤φVc

For slabs and floor joists, it is advisable to increase the cross-section of the element under consideration rather than add shear reinforcements.

Design of Shear Reinforcement

Note that shear force may vary along the length of the beam so the design must be based on the most critical value. The spacing of the links may then be increased as the shear value decreases keeping the same link diameter and number of legs.

ACI code requires (cl.11.5.7.2)

Vu≤φVn

Vn=Vc+Vs

By considering a free-body diagram along a diagonal crack intercepting n-stirrup leg each with a cross-section area of Av,

the shear capacity of the reinforcement is given by

Vs=Avfyn

where			n=ds

Vs=AvfydsAvs=Vsfyd=(Vn−Vc)fyd

Critical Section for Nominal Shear strength

- For gradually varying shear force, the critical section is evaluated at a distance d from the face of the support which coincides with the first inclined crack. The design shear reinforcement must be extended into the support.

- The critical section should be taken at the face of the support in the following cases (cl. 11.13):

- When the support is itself a beam or girder and therefore does not introduce compression into the end region of the member;

- When a concentrated load occurs between the face of support and the distance d from the support.

- When loading may cause a potential inclined crack to occur at the face of support and extend into instead of away from the support.

Other Code requirements:

- To prevent shear-compression failure, the code limits the diagonal compression stress produced by shear below the compressive strength of concrete. Hence

Vs≤2f'c3bwd

Where this is not satisfied, beam cross-section must be increased.

- Since the derivation requires that one or more stirrups cross the diagonal crack, limits are placed on the stirrups spacing (cl.11.5.5):

  - if        Vs≤f'c3bwd     then        smax ≤ d/2 ≤ 600mm

  - if        Vs≥f'c3bwd      then           smax ≤ d/4 ≤ 300mm

- Minimum links (cl.11.5.6.3):

Av,min=0.062fc'bwsfy 0.35bwsfy

- Stirrups must be anchored by running them into the compression zone and adding hooks or bending them around main steel.

Bond and Anchorage

The development length concept is based on the attainable average bond stress over the length of embedment of the reinforcement. Development lengths are required because of the tendency of highly stressed bars to split relatively thin sections of restraining concrete.

A single bar embedded in a mass of concrete should not require as great a development length; although a row of bars, even in mass concrete, can create a weakened plane with longitudinal splitting along the plane of the bars.

- Bond stresses are shear-type stresses created between tensile rebar and concrete which allow both materials to undergo the same deformations at that point with no slippage.

The value of these stresses must be limited to avoid crushing or splitting of the concrete in that region leading to slipping of the rebar, loss of composite action and failure of the beam.

Bond stresses vary along the length of the beam due to:

- Rate of Variation in bending moment (shear force)

- Formation of tension cracks

Hence, rather than calculate the value of bond stressing the bar, the available development length measured from the point of maximum stress in the bar to the point of zero stress (e.g. end of the bar) with the minimum length required for assured anchorage.

Factors influencing bond strength:

- Chemical Adhesion: limited in strength (up to 2MPa) in lightly stressed bars; broken by slightest slippage between concrete and steel.

- Friction: good except when bars are epoxy-coated for corrosion.

- Bearing: of bar ribs or lugs against concrete is the main contributor to bond strength. These bearing stresses are inclined at angle of 45o-80o to the longitudinal axis of the bar. Bond failures are initiated by the radial component of these stresses.

Failure Modes:

Bearing failure modes 1 & 2 depend on the smaller of the clear bottom cover cb and half the clear spacing between adjacent bars or the side cover cs.

Mode 1: Side-split crack: If bottom cover is large but side cover is not or the bars are closely spaced, splitting initiates along a horizontal plane extending through the row of reinforcements.

Mode 2: v-notch failure: If bottom cover is too small, longitudinal cracking initiates as diagonal tension or flexural cracks which lengthen and join together to form a continuous crack destroying the bond between concrete and steel resulting in spalling of the concrete.

Mode 3: Pullout Failure: If both cover (≥ 2.5) and spacing of bars(≥ 5) are large, bond failure occurs by pullout of the bar when the concrete between the ribs crush or shear off. It is most critical in weak or porous concrete.

Consequently, bond strength is reduced (by about 30%) in top bars compared to bottom bars (why?)

Stirrups moderately increase bond strength if positioned to cross failure plane produced by splitting.

Nominal Bond Strength

The three types of tests to evaluate the bond strength include the pull-out test, embedded rod test and the beam test. The last two better represent actual beam behavior. It can be established that

u=kfc' ,                         where k is a constant

assuming shear-type uniform bond stress u developed along the bar's surface over the development length, ld, the latter being measured from the point of maximum stress in the bar to the point of zero stress.

The bar force to be anchored, dT, is given by

dT=uπdblddT=πdb24fs

Hence, the average bond stress

u=fsdb4ld

and the development length

ld=fs4udb

If bond strength corresponds to the yield strength of the bar then the basic development length ldb is given by

ldb=k1dbfyfc'

Tension Bars (cl.12.2)

ACI replaces the constant in the above equation by multipliers that take into account other factors which influence bond strength such that

lddb=fy1.1fc'ψtψeψsλcb+ktrdb(As required)/(As provided)

| Constant | Significance | Limiting Values |
| --- | --- | --- |
| cb | smallest of the side cover measured to the center of the bar or 1/2 the center-to-center spacing of the bars |  |
| Ktr | represents the confining reinforcement across potential splitting planes and is given by: ktr=Atfyt10sn n is the number of longitudinal bars developed along the plane of splitting | 1.5≤cb+ktrdb≤2.5 Upper limit eliminates pullout failure |
| ψt | reinforcement location factor to reflect the adverse effects of the top reinforcement | 1.3 for top bars 1.0 for bottom bars |
| ψe | effects of epoxy coating | - if uncoated 1.2-1.5 for coated |
| ψs | reinforcement size factor | 0.8 for ф ≤ 20mm 1.0 for ф > 20mm |
|  | reflects the lower tensile strength of lightweight concrete | - for NWC 1.3 for LWC |

Where reinforcement detailing is in accordance with ACI requirements, the following simplified terms can be used for NWC for uncoated bars:

|  | ф ≤ 20mm | ф > 20mm |
| --- | --- | --- |
| Clear spacing of bars being developed or spliced not less than db, clear cover not less than db, and stirrups or ties throughout ld not less than the code minimum or Clear spacing of bars being developed or spliced not less than 2db and clear cover not less than db andcb+ktrdb=1.5 | lddb=fyψt2.1fc' (38 db) | lddb=fyψt1.7fc' (47 db) |
| Other unfavorable conditions cb+ktrdb=1.0 | lddb=fyψt1.4fc' (57 db) | lddb=fyψt1.1fc' (73 db) |

If ld > available anchorage length a hook is added to the end bar or the bar end is welded to a steel plate.

Compression Bars (cl.12.3)

Lack of cracking and end-bearing of the bar against the concrete reduce the development length required to anchor compression bars.

For deformed bars,

ldc=0.24fyfc'db≥0.043fydb(As required)/(As provided)

Bundled Bars (cl.12.4)

Development length of individual bars within a bundle, in tension or compression, shall be that for the individual bar increased by:

- 20% for three-bar bundle

- 33% for four-bar bundle

For determining the appropriate factors, a unit of bundled bars shall be treated as a single bar of a diameter derived from the equivalent total area.

Standard Hook in Tension (cl.12.5)

- Hooks are added to provide additional anchorage capacity when required development length cannot be achieved in tension bars (only).

- The development length ldh is measured from the critical section (max stress) to the outside end (or edge) of the hook.

- Both 90 degree bends and 180 degree bends can be used (check detailing requirements for min. radius and extension)

ldh=0.24fyfc'dbAs,requiredAs,provided³8db³150mm

- Modifiers include:

| Conditions | Modifier |
| --- | --- |
| Hooks with db 36mm with side cover  65 mm or (50 mm and for 90 degree hook) | 0.7 |
| Hooks of db 36mm enclosed by stirrups with s 3db along ldh such that first stirrup enclose the bent portion of the hook, within 2db of the outside of the bend. | 0.8 |

Requirements Cut-off bars

- In beam design, areas of steel are specified for zones of maximum positive and negative moments. It is undesirable to extend bars along the full-length of beam for economic reasons as well to relieve steel congestion.

- Theoretical cutoff points can be specified to reduce a specific percentage of steel to 40-50% of the originally specified.

- All cut-off bars must be extended by a distance d or 12db beyond the cut off point to provide for shifts in the location of maximum moments.

- Shear requirements at Cutoff points:

Reduced shear strength and loss of ductility are observed when bars are cut off in the tension zone. Flexural reinforcement shall not be terminated in a tension zone unless one of the conditions below is satisfied:

- At cutoff, Vu≤23fVn

- Additional stirrup Ast (in excess of that required for shear and torsion) is provided along each terminated bar, over a distance ¾ d from the termination point such that:  Asts=0.42bwfyt

Where s   d/(8βb) and βb is the ratio of the steel area terminated to the total steel at cut-off.

- For db  36mm, continuing reinforcement provides double the area required for flexure at the cutoff point i.e. As,provided  ≥ 2As,required and Vu≤34fVn

ACI Requirements for Positive Steel (cl.12.11)

- At least 1/3 the positive moment reinforcement in simple members and 1/4 the positive moment reinforcement in continuous members must extend into the support by at least 150 mm.

- Bar diameters must be limited in size such that:

ld≤MnVu+la

where Mn is nominal capacity of the section based on the continuing steel, assuming all reinforcement stressed to fy;

Vu is shear force at the cut off section;

la at a support is the embedment length beyond center of support but at a point of inflection, it is limited to the greater of d or 12db

- An increase of 30% is permitted when the ends of reinforcement are confined by a compressive reaction (e.g. at simple supports)

- If the above condition are not satisfied, then the following alternatives may be considered:

- using smaller bars (to lower required ld)

- using hooks instead at simple support

- allowing more steel to continue (increase section capacity, Mn)

ACI Requirements for Negative Steel (cl.12.12)

- At least 1/3 of the total tension reinforcement provided for negative moment at a support shall have an embedment length beyond the point of inflection not less than d, 12db, or ln/16, whichever is greater.

Splicing of Reinforcement Bars

Reinforcing bars come in specific lengths but can be joined together to ensure continuity where longer lengths are required e.g. columns.

Bars can be spliced together by:

- Welding

- Mechanical connectors

- Lap splices in which bars are extended past each other far enough to permit the force in one bar to be transferred by bond stress through the concrete to the other bar provided space between bars is limited to s ≤1/5 ls or 150mm whichever is smaller.

The last is the cheapest and the most common but results in congestion and may initiate transverse cracks at splice ends due to stress concentration.

Tension Lap

- Class A splices are allowed when:

- the area of reinforcement provided is at least twice that required by analysis over the entire length of the splice; and

- one-half or less of the total reinforcement is spliced within the required lap length

- Minimum length of lap for tension lap splices are classified as Class A or B splice, but not less than 300 mm, where:

Class A splice =1.0ld, when certain conditions apply

Class B splice =1.3ld, when class A is inapplicable

where ld is calculated to develop fy without modifiers of (12.2.5).

Compression Lap

Compression lap splice length is given by (but not less than 300 mm):

- 0.071fydb, for fy  420 MPa

- (0.13fy – 24)db for fy > 420 MPa,

- for fc′ < 21 MPa, lap splice must be increased by one-third.

Axially Loaded Short Columns

Columns are vertical compression members intended to support load-carrying beams and transmit the loads from all floors to the soil through the foundation.

Failure of one column in a critical location may lead to progressive collapse if the structural system fails to redistribute the load to other parts of the structure (viable alternative load path) without over-loading other members. As such, more reserve strength is required by the code.

Columns may carry axial load as well as bending moments due to frame action or eccentricity in the applied load. However, the axial load remains the dominant force in their design.  The ratio M/P defines the eccentricity in the column which can be ignored if it is <0.1h.

Classification of Columns

- According to lateral reinforcement bars which provide confinement of the concrete under compression and improves its ductility:

- Tied columns

- Spiral columns

- According to Loading

- Concentrically loaded columns: Axial load only, (Px) e.g. interior columns in frames with equal spans.

- Uniaxially loaded columns: Axial load and moment about one axis (Px, My) e.g. façade o edge columns

- Biaxially loaded columns: Axial load and moments about both axes (Px, My, Mz) e.g. corner columns

- According to Slenderness ratio ()

Slenderness ratio depends on the unsupported length of the column, the radius of gyration (minimum) and the column restraint at the joints. It dictates the failure mode of the column and its load carrying capacity.

- Shot columns: material failure by crushing of the concrete

- Long Columns : stability failure by buckling at a much reduced stress

Design of Rectangular Concentrically Loaded Columns

For each column, the tributary area at each level is evaluated and multiplied by the distributed dead load and live load for that level. The total load on a column at any level is the cumulative sum of the dead loads and the reduced live loads from all the levels above it.

Live Load reduction factors are as follows:

| Storey No. | Reduction factor |
| --- | --- |
| 1 | 1.0 |
| 2 | 0.9 |
| 3 | 0.8 |
| 4 | 0.7 |
| 5 | 0.6 |
| 6 or more | 0.5 |

Assumptions:

- uniform stain of 0.003 at failure

- concrete maximum capacity of 0.85f’c

- steel has yielded in compression

- minimum eccentricity ≤0.1h

Po=0.85fc'Ag+AsfyPn=0.8PalignloPu≤φPn

Where  for tied columns

Code Requirements:

- Minimum longitudinal reinforcement ratio of 1% to provide ductility, capacity for minimum eccentricity

- Maximum of 8% otherwise redesign the section

- Typical between 2-4% for economy and to avoid rebar congestion especially at beam-column junction

- Minimum tie diameter of 10mm for bar diameters < 32mm. For large bars, use 12mm diameter ties

- Longitudinal bars spaced more than 15mm apart must be supported by a tie.

- Spacing of ties should be the smaller of

- 48 ties diameter

- 16 long. Bar diameter

- Minimum dimension of the column

Foundation Systems

Foundation systems transmit safely the high concentrated column and wall vertical and lateral loading to the ground without overstressing the soil or causing intolerable (differential) settlements which will compromise the structural performance. The latter depends on the rigidity of the foundation system and the bearing capacity of the soil.

Types of Foundation

- Wall footing is a continuous strip along the length of the wall with greater width than the wall thickness. Main reinforcement is placed normal to the wall direction in the bottom layer. Most critical section is at the face of the wall.

- Isolated footing is rectangular or square in plan, located under single column. It is reinforced in both directions in the bottom layers. It is economical in soils with reasonable bearing capacity and for small column load.

- Combined footing supports 2 or more columns where isolated footings would result in overlap. They require top and bottom reinforcement.

- Raft or mat foundation is ideal in soils with low allowable bearing capacity for vey large depths. The rigidity of the raft reduces differential settlement.

- Piles transfer column loads by end bearing as well as friction along its length. They are either precast driven piles or bored cast in-situ piles. Concrete pile caps are required to ensure load transfer from the column to the pile(s). It is appropriate where soil profile reveals stronger strata at higher depth.

- Piled raft is a combination of (4) and (5) above.