SI main  units :

Quantity                    Unit                  Symbol

length                           meter                     m

weight                          kilogram                Kg

time                             second                    sec

current                         amper                     A

Tempreture                  keiven                    K

illumination                 candel                    S

SI derived units :

Quantity                           unit                      Symbol

frequency                          hertz                       Hz

force                                  newton                   N

pressure                             pascal                     Pa

power                                watt                        W

electric charge                  coulomb                  C

electric potential               volt                         V

electric resistance             ohm                        Ω

electric conductance         siemens                  S

magnetic flux                   weber                     Wb

magnetic flux density       tesla                       T

luminous flux                   lumen                     lm

illuminance                      lux                          lx

Work/energy                    Joel                        J

Speed                               meter/second          m/sec

Density                            Kg / cubic-mete      Kg/m3

Volume                            cubic-mete              m3

Area                                 square meter            m2

multiples and sub-multiples of SI units:

| Factor | Symbol | Prefix | Factor | Symbol | Prefix |
| --- | --- | --- | --- | --- | --- |
| 10-1 | d | deci | 1024 | Y | yotta |
| 10-2 | c | centi | 1021 | Z | zetta |
| 10-3 | m | milli | 1018 | E | exa |
| 10-6 | µ | micro | 1015 | P | peta |
| 10-9 | n | nano | 1012 | T | tera |
| 10-12 | p | pico | 109 | G | giga |
| 10-15 | f | femto | 106 | M | mega |
| 10-18 | a | atto | 103 | K | kilo |
| 10-21 | z | zepto | 102 | H | hecto |
| 10-24 | y | yocto | 101 | Da | deca |

Definitions

Ampere: (A)

it’s the a charge of one coulomb passing through a point for one second

Volt: (V)

It’s a potential difference between two points  which allows the flow of one ampere current

for one watt power.

Ohm(Ω)

it’s the amount of resistance between two points which allows the flow of one ampere of current when one volt applied between them

Components of electric circuit:

1-source: It may be a battery or a generator.

2-conductors: to transmit current

3-control(s): which allows or breaks the current.

4-load: any item which current passes through

cell

switch

lamp

wires

Main Factors effecting resistance

1- Area (A):Its inversely Proportional to the area measured in m2

Rµ1/A.     ……………..(1)

2-Length (l):

Its directly Proportional to the length measured by meter

3- Resistivity :  Its directly Proportional to the Resistivity measured by Ω.m

R=ρℓAFrom equations 1,2 and 3 we get:

…………….(4)

Example :

Calculate the resistivity of a metal wire of cross-section diameter 0.4mm if a 25cm length of this wire has a resistance of 6Ω

A = π d2/4 =     = π x (4 x 10 - 4 m)2 / 4   =   π x (1.6 x 10 - 7 m2) / 4  =   1.2566 x 10 - 7 m2

ρ = RA/L  =   (6 Ω) x (1.2566 x 10 - 7 m2 ) / (0.25 m)    =  = 3.02 x 10 – 6   Ω.m

Temperature Effects On resistor value:-

Resistance increases almost linearly with an increase in temperature to the inferred absolute temperature of  ̶ 234.5 C

Let R0  be the resistance at 0 C°

Let Rt  be the resistance at  t C°

ΔR = Rt - R0                    ……...…………5

ΔR µ R O. t                      …….………….6

ΔR  = µ 0 R O. t             …………………7

µ 0  is constant called temperature coefficient  at 0C°

From 5 and 7 we get:

Rt - R0= µ 0 R 0. t

Rt = R0 ( 1+ µ 0 t)

…………..8

Rt = R0 ( 1+ µ 0 t)           ……………….

Temperature coefficient at different temperatures:Temperature coefficient changes when

temperature changes using the following formula

µ 0

µ t =   _________________            ……………..9

1 +   µ 0  t

Resistor at two different temperatures

Let R1  be the resistance at t1 C°

Let R2  be the resistance at t2 C°

R1   =  R0(1+ µ 0 t1)                            ……...…………10

R2  =  R0(1+ µ 0 t2)                          …….………,,….11

R1             R0(1+ µ 0 t1)

12    ……………………             ــــــــــــــــــــــــ       =   ـــــــ

R2             R0(1+ µ 0 t2)

R1             R0(1+ µ 0 t1)

…………………13              ــــــــــــــــــــــــــــــــ      ------     =    ـــــــــــــ

R2             R0(1+ µ 0 t2)

Definitions:

Resistance( R ) : The ability of conductors  to oppose the flow of electric current  is known as resistance,  measured in ohm  Ω

Conductance(G):

it’s the The ability of conductors to allow the flow of electric current, it is reciprocal of resistance R, Measured by mho( U )

G= 1/R

Resistivity (ρ): it’s the property  of conductors  for resistance  .

Measured by  ohm meter (Ω.m)

Conductivity(σ): It’s the property  of conductors  for conduction . It’s reciprocal of resistivity(ρ) , measured by    U /m

Example1:

A Resistor 36Ω at 20 C°, changed to 40 Ω at 50 C°, calculate the following:-

1- temperature coefficient at 0 C° ,

2- temperature coefficient at 100 C°,

3- resistor at 0 C°?

R1= 36Ω,  R 2 = 40 Ω ,t1= 20 C° , t2= 50C°

R1           R0(1+ µ 0 t1)

ــــــــــــــــــــــــــــ   = ـــــــ

R2          R0(1+ µ 0 t2)

36         (1+ µ 0x 20)

ـــــــــــــــــــــــــــــــــــــــــ     =  ـــــــــــــ

40         (1+ µ 0x 50)

µ 0      =  0.0065  1/ C°

µ100    =  0.0038  1/ C°

R0  = R1/(1+ µ 0 t1)= R2/(1+ µ 0 t2)

=  36/(1+0.0065x20) =31.8 Ω

Example2:If a resistor is 36Ω at 0C° find its value a  50 C° if the temperature coefficient is
               2.22x10-3   1/C°,

R0  = 36 Ω, µ0 = 2.22x10-3  1/ 0C° , t  = 50 C° ,Rt  = R20 = ?

Rt = R0 ( 1+ µ 0 t)

R20 = 36(1+ 2.22x10-3 x 50)

R20  = 40 Ω

Example3: If the temperature coefficient of resistor is 0.005  1/C° at  0 C°  find temperature coefficient 20C°:

t=20 C° , µ 0 = 0.005  1/C° ,  µ 20 = ?

µ t     = µ 0 /(1+ µ 0 t), µ 20  = µ 0 /(1+ 0.005x 20)  =0.0045  1/ C°

Ohms law

The ratio of voltage V and current  I  in a closed circuit is constant called resistance R.

V=I R , I = V/R  , R = V/I

V

R

V: is the potential difference

R

(voltage) in volts

I : Current  in amperes (A)

R : resistance in ohm Ω

I

From Ohm’s Law we get

:

Series connection of resistors: Two elements are in series if their currents are same.

R2

R1

Series

R2

R1

Not Series

Parallel connection :Two resistors are in parallel if they are (share) the same voltage.

Not Parallel

Parallel

R1

R2

R1

R2

Series connection

Vt = V1 +V2 +V3

IRt = IR1+IR2+IR3

IRt = I( R1 + R2 + R3)

Rt = ( R1 + R2 + R3)

Parallel

It = I1 +I2 +I3

V/Rt = V/R1+V/R2+V/R3

V/Rt =V(1/R1+1/R2+1/R3)

1/Rt = 1/ R1 + 1/R2 +1/ R3

Parallel connection

Series connection

Y to D Transformation

- There are TWO type of transformations which are Delta (D) to star connection (U) and vice versa

star-Delta Transformations& star-Delta Transformations:

Delta -> Star

Star -> Delta

Example: transform three delta connected resistors 6, 12, 18 Ω to star

12Ω

18Ω

6Ω

R3=18x66+12+18=3Ω

xample: transform three star connected resistors 2,3,6Ω to delta

3Ω

2Ω

6Ω

Type of sources

RS

انواع المصادر

مصدرفولتيه

1- Voltage sources:

It is composed of:

a - Voltage of the source VS

VS

b- Internal resistance RS in series

مصدرتيار

2-current  source  :

It is composed of:

a- Current of the  source IS

RS

IS

b- Internal resistance RS in parallel.

تحويل(تبديل)المصادر

Source transformation :

I

s

R

s

V

+

_

I

Load

+

_

V

S

R

S

V

+

_

I

Load

RL

RL

I = IS RS/(Rs+RL)                                                I = VS / (Rs+RL)

Condition for source transformation is I is same:

Vs/(RS+RL) =  IS *RS / (RS+RL)           OR   VS = ISRS

Maximum power transfer:

نقل اعظم قدره للحمل

+

_

V

S

R

S

V

+

_

I

Load

I = Vs/(Rs+RL)   ,    P=I2 RL= [Vs/(Rs+RL)]2 RL  ,   for maximum power dP/dRL= 0

dP              Vs RL*2(Rs+RL) – (Rs+RL)2* Vs

=

=

0.

.

dRL                                (Rs+RL)4

(Rs+RL)4   ≠  0. Result will be infinity (unknown)

Vs RL*2(Rs+RL) – (Rs+RL)2* Vs = 0.      we get

RS=RL

Condition of maximum power external resistance RL  = internal resistance RS

للحصول على اعظم قدره يجب ان تتساوى كل من المقاومة الخارجية RL و المقاومة الداخليه RS

example1:

For the circuit use source transformation to change: 1- current to voltage 2-   voltage to current

2Ω

6Ω

3Ω

12V

24V

3A

12V

24V

6Ω

2Ω

18V

6A

2Ω

6Ω

3A

8A

3Ω

3Ω

1-current to voltage Vs = Is Rs = 3x6 = 18V

2-voltages to current source Is1 = Vs /Rs = 12/2=6A,Is2 = 24/3=8A

example2:

For the circuit  find RS for maximum power transfer  if the load resistance is

A:  2Ω                             B : 3Ω                                  C: 6Ω

Answer:

A: making  all sources inoperative (voltage sources short circuit current sources open circuit)

then we remove 2 Ω from any circuit RS is=total resistance between 2 Ω

6Ω

3Ω

A- RS =( 3x6) /(3+6) = 3 Ω

B: making  all sources inoperative (voltage sources short circuit current sources open circuit)

then we remove 2 Ω from any circuit RS is=total resistance between 2

6Ω

2Ω

B- RS =( 2x6) /(2+6) = 3/2= 1.5 Ω

C making  all sources inoperative (voltage sources short circuit current sources open circuit)

then we remove 2 Ω from any circuit RS is=total resistance between 2

2Ω

2Ω

C- RS =( 2x3) /(3+2) = 6/5=1.2 Ω